AMD GPU Cycle Simulator
Lightweight Cycle Simulator Design Document

For Compiler and Operator Library Optimization Analysis

1. Project Overview

In GPU compiler and operator library development, performance analysis and optimization are critical components. Traditional performance analysis tools often provide limited visibility at the hardware level, making it difficult to perform detailed performance characterization at the instruction level. This project aims to develop a lightweight AMD GPU cycle simulator to provide precise performance data for compiler optimization and operator library tuning.

Unlike traditional RTL-level hardware simulators, this simulator uses behavioral-level modeling, pursuing a balance between simulation speed and memory efficiency while ensuring functional correctness. This design enables the simulator to handle real-world operator libraries and applications while providing sufficient performance insight capabilities.

2. High Level Design

2.1 Overall Architecture

The simulator's architecture adopts a layered design approach:

• Host Execution Layer: Executes host code on CPU, handles memory transfer and synchronization

• Runtime API Layer: Intercepts GPU API calls, simulates device memory allocation and deallocation

• Wave Scheduling Layer: Creates waves based on kernel configuration, manages wave allocation and scheduling

• Instruction Execution Layer: Executes waves instruction by instruction, maintains PC and register state

• Resource Model Layer: Models execution unit throughput and latency

• Trace Recording Layer: Perfetto integration for recording execution timeline and statistics

2.2 Input/Output Specifications

2.2.1 Input Format

The simulator accepts the following inputs:

• HIP/CUDA compiled .out executable files (ELF format)

• Device code binaries containing GPU instructions

• Kernel metadata (grid/block dimensions, shared memory size, etc.)

2.2.2 Output Format

The simulator produces the following outputs:

• Overall Cycle Statistics: Total program cycle count and breakdown by kernel

• Wave-Level Statistics: Per-wave execution statistics including IPC, stalls, and unit utilization

• Execution Bubble Visualization: Visual representation of instruction queue and execution unit idle states

• Perfetto Trace: Timeline visualization of cycle execution

2.3 Core Conceptual Model

2.3.1 Wave Execution Model

AMD GPUs use SIMD (Single Instruction Multiple Data) execution model, where each wave contains 64 threads. In CDNA architecture, waves are the basic scheduling units executing on SALU (Scalar Execution Unit) and VALU (Vector Execution Unit).

3. Low Level Design

3.1 Module Architecture

The simulator uses the following module structure:

gpu_cycle_simulator/

├── core/                    # Core simulation engine

│   ├── simulator.h          # Main simulator class

│   ├── wave_executor.h      # Wave executor

│   ├── instruction_decoder.h # Instruction decoder

│   ├── register_file.h      # Register file

│   └── memory_system.h       # Memory system

├── scheduler/               # Scheduling related

│   ├── wave_scheduler.h     # Wave scheduler

│   ├── cu_scheduler.h       # CU-level scheduling

│   └── dispatch_unit.h      # Dispatch unit

├── models/                  # Hardware models

│   ├── execution_unit.h     # Execution unit model

│   ├── pipeline.h           # Pipeline model

│   ├── memory_hierarchy.h   # Memory hierarchy model

│   └── cu_model.h           # CU model

├── runtime/                 # Runtime interception

│   ├── hip_hook.h           # HIP API Hook

│   ├── memory_manager.h     # Memory manager

│   └── kernel_launcher.h    # Kernel launcher

├── trace/                   # Trace and statistics

│   ├── trace_logger.h       # Trace recorder

│   ├── statistics.h         # Statistics collector

│   ├── perfetto_writer.h    # Perfetto output

│   └── report_generator.h   # Report generator

└── utils/                   # Utilities

├── elf_parser.h         # ELF parser

├── isa_tables.h         # ISA definition tables

└── config.h             # Configuration management

3.2 Core Data Structures

3.2.1 Wave Descriptor

struct WaveDescriptor {

uint32_t wave_id;           // Global Wave ID

uint32_t cu_id;             // CU ID

uint32_t pc;                // Program Counter

uint32_t sgpr_start;        // SGPR start index

uint32_t vgpr_start;        // VGPR start index

uint32_t mask;              // Active thread mask (64-bit)

WaveState state;            // Current state

uint64_t start_cycle;       // Start cycle

uint64_t active_cycles;     // Active cycle count

std::vector<Instruction> instr_stream;  // Instruction stream

std::queue<StallEvent> stall_history;  // Stall history

};

3.3 Simulation Flow

3.3.1 Main Simulation Loop

void Simulator::run() {

initialize();

while (!is_done()) {

uint64_t cycle = current_cycle();

process_completed_operations(cycle);

schedule_ready_waves(cycle);

execute_active_waves(cycle);

update_execution_units(cycle);

collect_statistics(cycle);

if (trace_enabled_) {

record_trace(cycle);

}

advance_cycle();

}

finalize();

generate_reports();

}

4. Perfetto Integration

4.1 Perfetto Data Model

Perfetto uses the Trace proto format and supports rich trace data. This simulator converts cycle simulation data into Perfetto-readable trace format:

• Timeline Events: Track wave execution, memory access, and barrier synchronization

• GPU Counter Track: Record execution unit utilization over time

• Performance Metrics: IPC, stall reasons, and bottleneck analysis

4.2 Timeline Visualization

DPC Execution Timeline (CU0, cycles 1000-1016):

┌─────────────────────────────────────────────────────┐

│ Cycle │ VALU │ VMEM │ LDS │ GDS │   Notes         │

├───────┼──────┼──────┼─────┼─────┼─────────────────┤

│  1000 │ ████ │ ░░░░ │ ░░░ │ ░░░ │ VALU active      │

│  1001 │ ████ │ ░░░░ │ ░░░ │ ░░░ │                   │

│  1002 │ ████ │ ████ │ ░░░ │ ░░░ │ VMEM start      │

│  1003 │ ░░░░ │ ████ │ ░░░ │ ░░░ │ VALU stall      │

└───────┴──────┴──────┴─────┴─────┴─────────────────┘

Legend: ████ = Active, ░░░░ = Idle/Stall

5. Output and Statistics

5.1 Overall Statistics Report

=============================================================

GPU Cycle Simulator Report

=============================================================

Program: matrix_mul_kernel

Total Cycles: 2,345,678

Total Waves: 1024

Average IPC: 0.67

-------------------------------------------------------------

Execution Summary

-------------------------------------------------------------

Kernel Statistics:

Kernel Name                    Cycles      %     Waves

─────────────────────────────────────────────────────────

matrix_mul_kernel              1,234,567   52.6   1024

softmax_kernel                   456,789   19.5    512

barrier_sync_kernel              234,567   10.0   2048

-------------------------------------------------------------

Resource Utilization

-------------------------------------------------------------

Execution Unit     Active Cycles   Total Cycles   Utilization

────────────────────────────────────────────────────────────

VALU Pipeline          1,200,000     2,345,678      51.2%

VMEM Pipeline            450,000     2,345,678      19.2%

LDS Pipeline            234,000      2,345,678      10.0%

SALU Pipeline           890,000      2,345,678      37.9%

=============================================================

5.2 Wave-Level Statistics

Wave Execution Details (Sample):

Wave ID    CU    PC Start    Cycles    IPC    Stalls    Unit Busy

─────────────────────────────────────────────────────────────────────

#001       CU0   0x1000      512      0.50     12       VALU: 45%

#002       CU0   0x1000      508      0.50     14       VALU: 44%

#003       CU1   0x1000      520      0.49     18       VMEM: 52%

Top 10 Most Frequent Stall Patterns:

1. Memory Latency Wait:     1,234 cycles (5 stalls)

2. VALU Pipeline Busy:        890 cycles (12 stalls)

3. LDS Port Conflict:          456 cycles (8 stalls)

4. Barrier Synchronization:    345 cycles (4 stalls)

5. RAW Hazard:                 234 cycles (15 stalls)

6. Trace Logging System

6.1 Trace Log Format

The simulator supports multiple levels of trace detail:

• SUMMARY: Only summary statistics

• KERNEL: Kernel-level statistics

• WAVE: Wave-level statistics

• INSTRUCTION: Instruction-level trace

• DETAILED: Complete detailed trace

6.2 JSON Format Output

{

"simulation_id": "sim_20240101_143052",

"config": {

"target": "cdna3",

"wavefront_size": 64,

"cu_count": 40

},

"statistics": {

"total_cycles": 2345678,

"total_instructions": 1567890,

"average_ipc": 0.669

},

"waves": [

{

"wave_id": 42,

"cu_id": 0,

"pc_start": "0x1000",

"cycles": 512,

"instructions": 256,

"stalls": [

{"type": "RAW_HAZARD", "cycles": 25},

{"type": "MEM_LATENCY", "cycles": 150}

]

}

],

"unit_utilization": {

"VALU": 0.672,

"VMEM": 0.453,

"LDS": 0.389,

"SALU": 0.761

}

}

7. Validation and Testing Strategy

7.1 Functional Verification

• Golden Reference Tests: Verify against kernels with known correct results

• Random Instruction Tests: Generate random instruction sequences for correctness

• Regression Tests: Compare simulation results across different versions

• Boundary Condition Tests: Test behavior with extreme grid/block configurations

7.2 Performance Verification

• Profiling Comparison: Compare cycle ratios with rocprof results

• Unit Tests: Verify throughput and latency models for each execution unit

• Memory Latency Verification: Compare latency distribution for actual memory access patterns

• Integration Tests: Run standard benchmarks (e.g., ROCm performance tests)

8. Appendix

8.1 Glossary

| Term | Full Name | Description |
| --- | --- | --- |
| Wave | Wavefront | Basic GPU execution unit with 64 threads |
| CU | Compute Unit | AMD GPU compute unit |
| VALU | Vector ALU | Vector arithmetic logic unit |
| SALU | Scalar ALU | Scalar arithmetic logic unit |
| LDS | Local Data Share | Local data sharing (shared memory) |
| SGPR | Scalar GPR | Scalar general purpose register |
| VGPR | Vector GPR | Vector general purpose register |
| IPC | Instructions Per Cycle | Instructions per cycle ratio |

8.2 References

• AMD CDNA 3 Architecture White Paper

• AMD RDNA 3 Instruction Set Architecture Reference Guide

• ROCm Documentation (rocmdocs.amd.com)

• HSA ELF Loader Specification

• Perfetto Trace Protocol Documentation

8.3 Configuration Example

# simulator_config.yaml

simulator:

target_architecture: cdna3

wavefront_size: 64

cu_count: 40

cu_simd_count: 4

execution_units:

valu:

throughput: 1

latency: 4

count: 4

salu:

throughput: 1

latency: 1

count: 1

vmem:

throughput: 0.25

latency: 200

count: 1

lds:

throughput: 1

latency: 20

count: 1

memory_hierarchy:

l1:

size: 32KB

latency: 4 cycles

l2:

size: 512KB

latency: 20 cycles

dram:

latency: 200-400 cycles

trace:

level: wave

output_format: json

perfetto_output: ./trace.perfetto-trace

由Minimax Agent AI生成